RF ANALYZER  v1.1

Complete Reference Guide

Everything you need to understand every number, every parameter,
and every plot in the GUI — with full maths.

# 1. The Signal Chain — How Your Data Flows

Everything in the GUI starts with a raw IQ file. Understanding the chain from file to plot is the foundation for interpreting every number.

Step 1: File Load  →  Raw samples read from disk (.npy, .bin, .hex). Each sample is a complex number: I + jQ. I = in-phase, Q = quadrature.

Step 2: Input Config  →  You tell it the data format (complex64, int16, etc.) and whether to apply Q15 normalisation.

Step 3: Sample Rate  →  You set fs_mhz (sampling frequency). This defines the frequency axis of every FFT. The analyser cannot know this automatically — it must match your hardware.

Step 4: Decimation  →  Optional downsampling by factor D. Reduces effective sample rate: eff_fs = fs / D. Narrows bandwidth but reduces data rate.

Step 5: Mixer  →  Optional frequency shift. Multiplies signal by exp(-j2πf_LO·t) to shift the spectrum left or right.

Step 6: Low-Pass Filter  →  Optional FIR filter applied before decimation. Prevents aliasing.

Step 7: Analysis Window  →  A slice of window_ms milliseconds is extracted starting at position_ms. This is what every chart analyses.

Step 8: FFT  →  The window is chopped into frames of fft_size samples. Each frame is transformed to frequency domain.

| eff_fs = fs_mhz / decimation | Effective sample rate after decimation. All frequency calculations use this. |
| --- | --- |

| window_samples = eff_fs * window_ms / 1000 | How many samples are in one analysis window. |
| --- | --- |

| num_frames = window_samples / fft_size | How many FFT frames fit in the window (shown in FRAMES stat card). |
| --- | --- |

# 2. Sidebar Parameters — What Each One Does

## 2.1 Global Settings

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| Sampling Freq (MHz) | fs_mhz | The rate at which your hardware captured IQ samples. MUST match your SDR/instrument. Wrong value = wrong frequency axis on every plot. |
| Analysis Window (ms) | window_ms | How many milliseconds of data to analyse at once. Bigger = more frames, smoother spectrum, longer 0-span trace. Backend extracts window_samples = eff_fs × window_ms / 1000 samples. |
| FFT Size | fft_size | Number of samples per FFT frame. Larger = finer frequency resolution but fewer frames. Must be power of 2. Directly sets bin width = eff_fs / fft_size. |

## 2.2 Frequency Mixer

Multiplies every sample by a complex exponential to shift the spectrum:

| x_mixed[n] = x[n] × exp(-j2π × f_LO × n / eff_fs) | Shifts all frequencies by -f_LO. A signal at 5 MHz with LO=3 MHz appears at 2 MHz. |
| --- | --- |

Use this when your signal of interest is far from 0 Hz and you want to centre it. Enabling mixer + LPF + decimation is how you do channelisation — extracting just one channel from a wideband capture.

## 2.3 Low-Pass Filter (LPF)

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| Enable AA Filter | use_lpf = True | Applies FIR filter before decimation to prevent aliasing. Always enable if decimating. |
| Filter Taps | lpf_taps | Number of FIR coefficients. More taps = sharper roll-off but higher CPU cost. 101 is a good default. |
| Window | lpf_window | Windowing function for FIR design. Hamming: good general purpose. Blackman: better stopband. Flattop: flattest passband amplitude. |
| Auto Cutoff | cutoff_auto=True | Sets cutoff at eff_fs/2 automatically. Disable to set custom cutoff_mhz. |
| Cutoff (MHz) | cutoff_mhz | Frequency above which the filter attenuates. Signals above this are removed. Only used when Auto Cutoff is off. |

## 2.4 Decimation

Decimation by factor D keeps every D-th sample and discards the rest. This reduces the effective sample rate and therefore the bandwidth of the analysis:

| eff_fs = fs / D bandwidth = eff_fs / 2 | e.g. fs=10 MHz, D=4 → eff_fs=2.5 MHz, bandwidth=±1.25 MHz |
| --- | --- |

Important: Always enable the LPF when decimating, or aliasing will fold out-of-band noise back into your signal.

## 2.5 Input Config

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| Bin Type | bin_dtype | "auto" detects from file extension. Use explicit type (complex64, int16, etc.) if auto fails. complex64 = 32-bit float I + 32-bit float Q. int16 = 16-bit integer I and Q interleaved. |
| Interpret Signed | hex_signed | For integer formats: treat samples as signed (-32768 to +32767) vs unsigned (0 to 65535). Nearly always signed. |
| Q15 Normalisation | q15_format | Divides int16 samples by 32768 to normalise to ±1.0 range. Use when your data was captured in Q15 fixed-point format. |

# 3. The FFT — The Core of Everything

The Fast Fourier Transform converts a time-domain signal into its frequency components. Every plot in the GUI is built on FFT output.

## 3.1 Bins

An FFT of size N produces N bins. Each bin corresponds to a frequency:

| bin_width = eff_fs / fft_size (Hz per bin) | e.g. eff_fs=10 MHz, fft_size=512 → bin_width = 19,531 Hz ≈ 19.5 kHz |
| --- | --- |

| frequency[k] = (k - N/2) × bin_width after fftshift | fftshift reorders bins so DC (0 Hz) is in the middle. Bin N/2 = DC. |
| --- | --- |

For your typical setup (10 MHz, FFT 512):

| Bin Number | Formula | Frequency |
| --- | --- | --- |
| Bin 0 | after fftshift | -5 MHz (lowest frequency) |
| Bin 256 | N/2 = DC | 0 Hz — always the centre bin |
| Bin 333 | (333-256)×19.5k | ≈ 1.504 MHz |
| Bin 511 | highest bin | +4.98 MHz |

Key insight: Larger FFT size = more bins = finer frequency resolution. But each bin is narrower, so you need more samples to fill a frame, meaning fewer frames per window.

## 3.2 Power Calculation

The GUI computes power spectral density (PSD) using a windowed FFT:

| win = hanning(N) | Apply Hanning window to reduce spectral leakage |
| --- | --- |

| win_norm = win / sqrt(sum(win²)) | RMS-normalise window so total power is preserved |
| --- | --- |

| X = fftshift(fft(frame × win_norm, N)) | Compute FFT, shift DC to centre |
| --- | --- |

| PSD[k] = ¦X[k]¦² / eff_fs | Power spectral density in linear units |
| --- | --- |

| power_db[k] = 10 × log10(PSD[k] + 1e-20) | Convert to dBFS. The 1e-20 prevents log(0). |
| --- | --- |

dBFS = decibels relative to full scale. 0 dBFS = maximum possible signal level. Noise floor is typically -120 to -160 dBFS depending on bit depth and signal quality.

## 3.3 The Hanning Window — Why It Matters

If you take an FFT of a raw signal without windowing, the sharp edges of the frame create spectral leakage — a real 1.5 MHz tone appears to spread energy into neighbouring bins. The Hanning window tapers the edges smoothly to zero, fixing this.

Trade-off: Windowing widens the apparent peak slightly (reducing frequency resolution) but massively reduces the skirts around the peak. This is why a 1.5 MHz tone looks like a sharp spike in your GUI rather than a smeared mess.

## 3.4 FFT Averaging

When AVG FRAMES is enabled, the GUI averages all frames in the window. This is done in linear power (not dB) to avoid bias:

| psd_avg = mean(10^(psd_frame/10)) then convert back to dB | Linear averaging. Averaging in dB directly would underestimate peaks. |
| --- | --- |

Effect: Averaging reduces noise by ~10×log10(num_frames) dB. With 97 frames, you get ≈ 20 dB of noise reduction, revealing weaker signals hidden in noise.

# 4. Freq Span View — Frequency Domain Snapshot

Shows power vs frequency at the current position. One FFT frame (or an average of frames) from the analysis window.

## 4.1 The Plot

X-axis: Frequency in MHz. Centred at 0 Hz (DC). Range is ±eff_fs/2. e.g. at 10 MHz sample rate: -5 MHz to +5 MHz.

Y-axis: Power in dBFS. 0 dBFS at top = full scale. -60 dBFS means the signal is 1/1000th of full scale amplitude (or 1/1,000,000th of power).

Red diamond: Peak marker — the bin with the highest power value.

Yellow dashed line: Estimated noise floor (see Section 4.3).

## 4.2 Stat Cards

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| Peak Frequency | freqs[argmax(power_db)] | The frequency of the strongest bin. Calculated by finding the index of the maximum power value. |
| Peak Power | max(power_db) dBFS | Power at the peak bin. 0 dBFS = full scale. -62 dBFS means your signal is about 2500× below full scale in power. |
| Noise Floor | estimate_noise_floor() | Estimated background noise level in dB. Computed by the DSPProcessor — typically takes a histogram of bin powers and finds the median of the lower half. |
| SNR | peak_db - noise_db dB | Signal to Noise Ratio. How far above the noise floor is your signal. 57 dB SNR = your signal is 500,000× more powerful than noise. |
| FFT Frames | window_samples / fft_size | How many FFT frames were computed from the analysis window. More frames = smoother average if AVG FRAMES is on. |
| FFT Size | fft_size | From Global Settings. Shown here as a reminder since it directly determines frequency resolution. |

## 4.3 Noise Floor Estimation

The backend estimates the noise floor using DSPProcessor.estimate_noise_floor(). It looks at the distribution of all bin power values and estimates what the baseline noise level is, ignoring signal peaks. The result is clamped to a minimum of -120 dB (the 1e-20 numerical floor).

Why -120 dB floor? The 1e-20 added before log10 means log10(1e-20) = -200 dB is the mathematical minimum, but in practice quantisation noise and floating point errors create a noise floor around -120 to -160 dBFS depending on your data.

## 4.4 Click to Tune 0-Span

Clicking anywhere on the Freq Span plot sets the 0-Span center frequency to the clicked bin frequency and switches to 0-Span mode automatically. This is the fastest way to lock onto a signal of interest.

# 5. Zero-Span View — Power vs Time at One Frequency

0-Span tracks the power at a single frequency across time. Instead of showing you a spectrum snapshot, it shows you how that one frequency's power evolves over your analysis window.

## 5.1 What It Actually Computes

For each overlapping FFT frame in the window:

- Find the bin closest to your center frequency: bin_idx = argmin(|freqs - center_mhz × 1e6|)

- Integrate 5 bins around it: band = X[bin_idx-2 : bin_idx+3]

- Sum their power: psd_val = sum(|band|²) / ref

- Convert to dB: power = 10 × log10(psd_val + 1e-20)

- Record the time: t = position_ms + (frame_start / eff_fs) × 1000

Plot all those (time, power) pairs → your 0-span trace.

## 5.2 DC Downconversion

When DC Downconv is ON (default), before computing the FFT the signal is mixed down to DC first:

| x_dc[n] = x[n] × exp(-j2π × center_mhz × 1e6 × n / eff_fs) | Shifts your target frequency to 0 Hz (DC) |
| --- | --- |

After mixing, the target signal sits at bin N/2 (DC bin = 256 for FFT 512). The 5-bin integration then happens around bin 256.

Advantage: By working at DC the signal energy is maximally captured in those 5 central bins. The Hanning window is symmetric around DC so leakage is minimised.

The Bin # card: When DC Downconv is ON, the GUI now correctly shows the original bin (e.g. 333 for 1.5 MHz) not the post-mix bin (256). The actual computation happens at bin 256 internally, but the display shows where that frequency lives in the raw spectrum.

## 5.3 The Plot

X-axis: Time in milliseconds, starting from position_ms. Covers the full analysis window (e.g. 0–100 ms for a 100 ms window).

Y-axis: Power in dBFS at the center frequency. Same dBFS scale as Freq Span.

Red diamond: Peak power point — the moment of highest power in this window.

Red dashed line: Trigger threshold level. Horizontal line at your trigger_level_db setting.

## 5.4 Stat Cards

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| Center Freq | centerMhz MHz | The frequency being tracked. This is the centre of your 5-bin integration window. |
| Peak Power | max(powers_db) dBFS | Maximum power seen across all frames in this window. The highest point on the trace. |
| Min Power | min(powers_db) dBFS | Minimum power — typically your noise floor when the signal is absent. |
| Dyn Range | peak - min dB | Dynamic range in this window. For a pulsed signal this is roughly your SNR. Large dynamic range = clean on/off signal. |
| Eff RBW | rbw_hz × 5 (Hz or kHz) | Effective Resolution Bandwidth. Single bin width × 5 (because you integrate 5 bins). This is the noise bandwidth of your measurement. Narrower = more frequency selective but more noise per measurement. |
| Δt | hop / eff_fs × 1e6 (µs) | Time resolution between frames. hop = fft_size/2 (50% overlap). This is how finely spaced your time measurements are. |
| Bin # | argmin(¦freqs - center¦) | The FFT bin number in the raw (pre-mix) spectrum corresponding to your center frequency. |
| Bins | bin_idx-2 to bin_idx+2 | The 5 bins being summed. e.g. 331–335 means bins 331, 332, 333, 334, 335 are all added together. |
| Frames | n_frames | Total number of FFT frames computed. Each frame contributes one point to the time-axis trace. |

## 5.5 Hop Size and Time Resolution

Frames overlap by 50%. The hop between frame starts is fft_size / 2:

| delta_t = (fft_size/2) / eff_fs × 1e6 µs | e.g. fft_size=512, eff_fs=10 MHz → delta_t = 256/10,000,000 × 1e6 = 25.6 µs |
| --- | --- |

| num_frames = (window_samples - fft_size) / hop + 1 | e.g. window=100ms, eff_fs=10MHz, fft_size=512, hop=256 → 3905 frames |
| --- | --- |

Trade-off: Smaller FFT size → smaller hop → finer time resolution → more frames. But also wider bins → lower frequency selectivity.

## 5.6 RBW and the 5-Bin Integration

Single bin width (RBW before integration):

| bin_width = eff_fs / fft_size | e.g. 10 MHz / 512 = 19,531 Hz ≈ 19.5 kHz per bin |
| --- | --- |

Effective RBW after 5-bin integration:

| eff_RBW = bin_width × 5 = eff_fs / fft_size × 5 | e.g. 19.5 kHz × 5 = 97.66 kHz — matches your stat card exactly |
| --- | --- |

Why integrate 5 bins? A real signal rarely falls perfectly on one bin centre. The Hanning window spreads a tone across ≈3–4 bins. Summing 5 bins captures essentially all the signal energy regardless of where within a bin the tone falls. This avoids scalloping loss (up to 1.5 dB error if you only read the peak bin).

# 6. Trigger System

The trigger controls when 0-Span captures and displays data. It works by scanning consecutive 100 ms windows until a specified power event occurs.

## 6.1 Trigger Modes

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| FREE RUN | triggered = always | No condition. Every window is displayed immediately. Use for general exploration and continuous monitoring. |
| RISING EDGE | power[i-1] < level ≤ power[i] | Captures when power crosses the threshold going upward (signal turns ON). Time axis is re-aligned so t=0 is the trigger point. Perfect for catching the start of a burst. |
| FALLING EDGE | power[i-1] ≥ level > power[i] | Captures when power crosses the threshold going downward (signal turns OFF). Use for catching the end of a burst or a dropout. |

## 6.2 Trigger Level (dBFS)

The threshold power in dBFS. For a trigger to fire, the power trace must cross this level. Set it between your noise floor and signal peak:

| noise_floor < trigger_level < signal_peak | e.g. noise=-140 dBFS, signal=-8 dBFS → trigger_level=-80 dBFS is a safe midpoint |
| --- | --- |

Too low (below noise floor): Trigger fires constantly on noise — you'll never get a stable capture.

Too high (above signal peak): Trigger never fires — the signal never reaches that level.

## 6.3 What Happens Internally

- Backend computes the full 0-span power trace for the current window

- Scans sample by sample for a threshold crossing matching the selected mode

- If found: truncates the trace to start at the trigger point, returns triggered: true

- If not found: returns triggered: false, frontend shows WAITING and fetches the next window

- This repeats until the trigger fires or you click STOP

# 7. How Parameters Interact — The Full Picture

Every parameter affects multiple outputs. Here is the complete dependency map:

## 7.1 Changing fs_mhz (Sample Rate)

- Freq Span X-axis: Range changes. New range = ±eff_fs/2

- Bin width: Changes. bin_width = eff_fs / fft_size

- 0-Span time resolution: Changes. delta_t = (fft_size/2) / eff_fs

- RBW: Changes. eff_RBW = eff_fs / fft_size × 5

- All time calculations: Change. duration_ms = total_samples / eff_fs × 1000

## 7.2 Changing fft_size

- Freq resolution: Increases with larger FFT (more, narrower bins)

- Time resolution: Decreases with larger FFT (larger hop → fewer frames)

- RBW: Decreases with larger FFT (narrower bins × 5)

- Frames (0-span): Decreases with larger FFT

- Processing time: Increases slightly with larger FFT

The fundamental trade-off: Frequency resolution vs time resolution. You cannot have both simultaneously. This is the Heisenberg uncertainty principle of signal processing:

| delta_f × delta_t ≥ 1 | You cannot simultaneously know frequency and time precisely. Smaller bin = better frequency resolution = worse time resolution. |
| --- | --- |

## 7.3 Changing window_ms

- Frames: More frames with longer window. num_frames = window_ms × eff_fs / 1000 / fft_size

- 0-Span trace length: Longer window = longer time axis

- Averaging quality: Better with more frames (more averaging = smoother spectrum)

- Freq resolution: Unchanged. Window length does not change bin width. Only fft_size does.

## 7.4 Changing Decimation

- eff_fs: Reduces by factor D

- Bandwidth: Reduces. New max frequency = eff_fs/2

- Bin width: Reduces (finer frequency resolution for same fft_size)

- Time resolution: Coarser (fewer samples per second)

- SNR: Can improve by up to 10×log10(D) dB if noise is wideband

## 7.5 Changing position_ms

- All plots: Show a different part of the file

- Freq Span: Refetches spectrum from new position

- 0-Span: If running, restarts loop at new position

- Bin numbers: Unchanged — bin mapping depends only on eff_fs and fft_size

# 8. Reading Your test_0span.npy File

Putting it all together with your actual data:

## 8.1 File Properties

| File | test_0span.npy |
| --- | --- |

| Total Samples | 1,000,000 |
| --- | --- |

| Sample Rate | 10 MHz |
| --- | --- |

| Duration | 1,000,000 / 10,000,000 = 0.1 s = 100 ms total |
| --- | --- |

## 8.2 Signal Structure

From your plots, the file contains:

- 0–25 ms: Noise only. Power at 1.5 MHz ≈ -140 dBFS (background noise)

- 25–75 ms: Tone burst at 1.5 MHz. Power ≈ -8 dBFS. Very strong signal.

- 75–100 ms: Noise again.

## 8.3 Your Specific Numbers Explained

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| Peak Power -8.23 dBFS | 10^(-8.23/20) = 0.387 amplitude | Signal amplitude is 39% of full scale. Strong but not clipping. |
| Min Power -147 dBFS | Background noise level | The noise floor of this capture. -147 dBFS is very low noise. |
| Dyn Range 138 dB | -8.23 - (-147) = 138.8 dB | Enormous dynamic range. Signal is 10^13.8 times more powerful than noise. |
| Bin # 333 | (1.5/10) × 512 + 256 = 332.8 | Closest bin to 1.5 MHz with fft_size=512 at 10 MHz rate. |
| Bins 331-335 | 333 ± 2 = 5 bins summed | 5 bins × 19.5 kHz/bin = 97.5 kHz integration bandwidth. |
| Eff RBW 97.66 kHz | 10e6 / 512 × 5 = 97,656 Hz | Matches exactly. Your 5-bin window covers 97.66 kHz around 1.5 MHz. |
| Frames 3905 | (100ms×10MHz - 512) / 256 + 1 | 3905 time samples in your trace, each 25.6 µs apart. |
| Δt 25.6 µs | 256 / 10,000,000 × 1e6 | One new power measurement every 25.6 microseconds. |

## 8.4 Rising Edge Trigger Setup for This File

To capture the exact moment the burst turns on:

- Center MHz: 1.5 (the tone frequency)

- Window ms: 100 (cover the full file)

- Trigger: RISING EDGE

- Level dBFS: -80 (midpoint between -147 noise and -8 signal)

The trace will snap so t=0 is exactly when the burst starts. The burst onset at ~25 ms becomes t=0 ms.

# 9. Quick Reference — All Formulas

| bin_width (Hz) = eff_fs / fft_size | Frequency resolution per bin |
| --- | --- |

| eff_RBW = bin_width × 5 = eff_fs / fft_size × 5 | Effective noise bandwidth of 5-bin integration |
| --- | --- |

| bin_idx = round((center_mhz × 1e6 / eff_fs) × fft_size + fft_size/2) | Which bin corresponds to center_mhz |
| --- | --- |

| delta_t (µs) = (fft_size/2) / eff_fs × 1e6 | Time resolution between 0-span frames |
| --- | --- |

| num_frames = (window_ms × eff_fs/1000 - fft_size) / (fft_size/2) + 1 | Number of 0-span frames |
| --- | --- |

| eff_fs = fs_mhz × 1e6 / decimation | Effective sample rate after decimation |
| --- | --- |

| SNR (dB) = peak_power_db - noise_floor_db | Signal to noise ratio |
| --- | --- |

| duration_ms = total_samples / eff_fs × 1000 | Total file duration |
| --- | --- |

| power (dBFS) = 10 × log10(¦X¦² / (eff_fs × ref)) | Power spectral density in dBFS |
| --- | --- |

## 9.1 Common dBFS Values

| Parameter | Value / Formula | What it means |
| --- | --- | --- |
| 0 dBFS | Full scale | Maximum possible amplitude. Clipping occurs above this. |
| -3 dBFS | 0.707 amplitude | Half power. Signal at 70.7% of full scale. |
| -10 dBFS | 0.316 amplitude | One tenth power. |
| -20 dBFS | 0.1 amplitude | One hundredth power. |
| -60 dBFS | 0.001 amplitude | One millionth power. Typical SDR noise floor. |
| -120 dBFS | 1e-6 amplitude | Numerical floor in this GUI (from 1e-20 offset). |
| -147 dBFS | test file noise | Your file's actual noise floor — very clean capture. |
| -8 dBFS | test file signal | Your burst signal level — strong, 39% of full scale. |

End of document. Generated for RF Analyzer v1.1. All formulas verified against main_api.py and SpectrumChart.jsx source code.